========================== FIRMWARE ==============================

POST:

+ CPU, RAM

+ peripherals: keyboard, mouse

+ devices: SSD, network adapter

Boot order configuration:

Usually -> Removable device (USB drive)

+ Look for a bootloader on bootable area (usually MBR)

========================== BOOTLOADER(GRUB) ==============================

Boot menu (optional: if multiple OS installed) -> user to choose:

Grub.cfg:

+ root (hd0, 0) – where compressed kernel + initrd image is located

+ ro root=/dev/VolGroup00/LogVol00 –

- where root filesystems (OS’s filesystems) - located

- Hey, kernel (after loaded) mount them read-only mode

- Loading kernel into executable zone (RAM – CPU accesses)

- Initrd into RAM (can be discared after that) as temporary root filesystems for kernel to mount them read-only mode

========================== KERNEL ==============================

+ Mount root filesystems in read-only mode

+ PID 1 - …